Search and find a best professional strategy, I am on raspberry/jetson and Pixhawk. Problem statement: I want to run a python that take a waypoint and to a given gps lat-long, after reaching at location, it should open another script, I have the way_point.py, and person_follow.py. Now give me strategy

Here’s a professional, production-style strategy to do:

- Fly to a target GPS (lat/long) using Pixhawk

- Detect arrival reliably

- Start person_follow.py automatically (and optionally stop/return later)
…on a Raspberry Pi / Jetson companion computer.

1) Make one “orchestrator” that controls the whole flow

Instead of “script A runs then script B runs”, run one supervisor (e.g., mission_manager.py) that:

- connects to the Pixhawk

- commands the waypoint navigation

- monitors distance-to-target

- when “arrived”, launches person_follow.py

This avoids fragile bash chaining and lets you handle failures, timeouts, mode changes, and clean shutdown.

2) Don’t let two scripts fight over the same MAVLink connection

This is the #1 real-world issue.

Best practice options

Option A (best): Only mission_manager.py connects to Pixhawk, and it imports your modules:

- way_point.py becomes a function/class you call

- person_follow.py becomes a function/class you call
✅ clean, single MAVLink connection, easiest to make reliable.

Option B (also solid): Run both scripts as separate processes, but use MAVLink Router so multiple clients can connect safely. MAVLink Router is specifically for distributing MAVLink packets to multiple endpoints.

3) Use a simple state machine (this is how pros do it)

In mission_manager.py implement states:

- INIT

- CONNECT (wait heartbeat, confirm GPS fix)

- SET_MODE_NAV

  - ArduPilot: GUIDED is the recommended dynamic control mode

  - PX4: use OFFBOARD (requires setpoint streaming)

- GOTO_TARGET

- ARRIVAL_CHECK (distance + speed + dwell time)

- START_FOLLOW (spawn person_follow.py)

- FOLLOW_RUNNING (monitor, fail-safe, exit conditions)

- END / RTL / LAND (optional)

This prevents weird edge cases (GPS jump, overshoot, drift).

4) Arrival detection: use “distance + dwell time + speed”

Don’t trigger “arrived” from distance alone. Use:

- distance_to_target <= R (e.g., 2–5 meters)

- groundspeed <= V (e.g., < 0.5 m/s)

- hold inside radius for T seconds (e.g., 3–5 seconds)

That eliminates false triggers due to GPS noise.

5) Mode and control recommendations (depends on firmware)

If you are on ArduPilot

- Use GUIDED and send a “go to” command (DroneKit or pymavlink). ArduPilot explicitly supports companion computer MAVLink control.

- Easiest: DroneKit simple_goto() or MAVLink MAV_CMD_NAV_WAYPOINT / guided position target.

If you are on PX4

- Offboard requires continuously streaming setpoints (commonly >2Hz, many people use 10Hz). PX4 Offboard docs explain the concept and requirements.

- Two common stacks:

  - MAVSDK offboard (clean API, async)

  - ROS/MAVROS offboard Python example exists (more “robotics platform” style)

If you’re unsure which firmware you’re on: Mission Planner usually means ArduPilot; QGroundControl often used for both.

6) How to launch person_follow.py safely

Launch it from the orchestrator using subprocess.Popen, and log stdout/stderr.

Also decide:

- Does person_follow.py need MAVLink control too?

  - If yes, prefer Option A (single connection) or Option B (mavlink-router).

- If follow script crashes, orchestrator should:

  - stop the drone (Loiter / Brake)

  - optionally retry follow

  - optionally RTL

7) Example “skeleton” flow (minimal but robust)

# mission_manager.py (conceptual skeleton)

import time, subprocess, math

TARGET_LAT = 24.8607

TARGET_LON = 67.0011

ARRIVAL_RADIUS_M = 3.0

DWELL_S = 4.0

MAX_TRAVEL_S = 300

def haversine_m(lat1, lon1, lat2, lon2):

R = 6371000.0

p1, p2 = math.radians(lat1), math.radians(lat2)

dphi = math.radians(lat2-lat1)

dlmb = math.radians(lon2-lon1)

a = math.sin(dphi/2)**2 + math.cos(p1)*math.cos(p2)*math.sin(dlmb/2)**2

return 2*R*math.asin(math.sqrt(a))

def main():

# 1) connect to Pixhawk (DroneKit/pymavlink/MAVSDK)

# 2) set mode GUIDED/OFFBOARD

# 3) command go-to TARGET_LAT/LON

start = time.time()

inside_since = None

while True:

# read current position + groundspeed from autopilot

# cur_lat, cur_lon, groundspeed = ...

dist = haversine_m(cur_lat, cur_lon, TARGET_LAT, TARGET_LON)

if dist <= ARRIVAL_RADIUS_M and groundspeed < 0.5:

if inside_since is None:

inside_since = time.time()

elif time.time() - inside_since >= DWELL_S:

break

else:

inside_since = None

if time.time() - start > MAX_TRAVEL_S:

# fail-safe: loiter/rtl

return

time.sleep(0.2)

# Arrived -> start follow

p = subprocess.Popen(["python3", "person_follow.py"])

# optionally monitor p.poll() and fail-safe on crash

if __name__ == "__main__":

main()

8) Deployment checklist (what makes it “professional”)

- Use SITL / simulation first

  - ArduPilot SITL or PX4 Gazebo/MAVSDK/MAVROS examples.

- Use systemd to auto-start mission_manager.py on boot

- Logging: save mission logs + events (mode changes, distances, timeouts)

- Health checks:

  - heartbeat timeout → go safe (Loiter/RTL)

  - GPS quality check before starting mission

- Resource control (Jetson):

  - pin CPU affinity / nice for vision process if needed

- Safety:

  - geofence + RTL configured

  - test with props off first (bench test)

  - clear “kill switch” mode on RC

9) My recommended “best” architecture for you

Given you already have way_point.py and person_follow.py:

Best path (clean + reliable):

- Refactor both into callable modules (WayPointController, PersonFollower)

- Build mission_manager.py orchestrator (single MAVLink connection)

- If you must keep them separate processes, add mavlink-routerd so both can connect safely.

If you tell me ArduPilot vs PX4, and how you connect (serial /dev/tty... or UDP), I can give you a concrete recommended stack (DroneKit/pymavlink vs MAVSDK vs MAVROS) and a tighter orchestrator template that matches your setup exactly.